User Commands                                        QuickPage(1)

NAME

QuickPage - SNPP client/server for sending  messages  to  an

alpha-numeric pager

SYNOPSIS

qpage [ -dhimQv ] [ -a [+][dd+]hhmm | -a YYMMDDHHMMSS ] [ -c

coverage  ]  [  -C  config  ]  [ -f from ] [ -l level ] [ -p

pagerid ] [ -P pagerid ] [ -s server ] [ message ]

qpage [ -d ] -q interval

DESCRIPTION

QuickPage sends messages to a paging terminal using the SNPP

and  IXO (also known as TAP) protocols.  It is normally used

with no options other than a recipient and the message text,

in  which  case the message is sent to the SNPP server where

it is submitted to a page queue to be  sent  by  a  separate

daemon  process.   Page  groups  and duty schedules are sup-

ported.  Status notification messages indicating the success

or  failure  of  a page are sent via e-mail to submitters of

high-priority (level 0) pages.

If no message is specified on the command line, the user  is

prompted  for  the  message  text.  Characters are read from

standard input until a line  containing  only  a  period  is

received.   All occurrences of whitespace within the message

text are reduced to a single space character.

Messages received by the QuickPage daemon  that  are  longer

than the maximum page length (see below) are split into mul-

tiple segments and sent as  separate  pages.   Each  message

segment  is  prefixed  with its segment number.  The maximum

number of segments a into which a message will be  split  is

configurable.   If  a  message  exceeds this limit, the last

allowed segment will be prefixed with 't',  indicating  that

the  message  is  truncated;  otherwise, the last segment is

prefixed with 'e' to indicate the end of the message.   Mes-

sages that fit into a single page are not prefixed with seg-

ment numbers.   All  messages  are  prefixed  with  CALLerid

information (see RFC-1861), if available.

OPTIONS

The following options are supported:

-a   Send the page at the specified time.  A time specifica-

tion  prefixed  with  a  plus  sign (+) is added to the

current time.  The QuickPage daemon will not  send  any

pages  with  a  timestamp  newer than the current time.

This feature may be used  to  provide  a  simple  alarm

clock function for meeting reminders, etc.  This option

applies only to the pagerid(s) specified by the next -p

option.

Thomas Dwyer III    Last change: 21 June 1997                   1

User Commands                                        QuickPage(1)

-c   Use a different coverage area or paging service.   This

option  is  only  useful if the recipient has more than

one pager and/or more than  one  paging  service.  This

option  applies only to the pagerid(s) specified by the

next -p option.

-C   Specify an alternate configuration file.

-d   Debug mode.  Very verbose and probably not  interesting

for  the  normal  user.   This  option  is provided for

debugging purposes only.

-f   Specify who the page is from.   This  option  specifies

the  argument to the SNPP CALLerid command.  Every mes-

sage segment will be prefixed with the value  specified

by this option.  A null argument ( -f "") may be speci-

fied to suppress the message prefix.   The  default  is

the userid of the person running QuickPage.

-h   Help.  Print  a  brief  summary  of  the  command  line

options.

-i   Use interactive mode.  The page is sent immediately and

summary  messages  are printed during the course of the

transaction.  This option may only be used on a machine

with  a  physically  attached  modem.   This  option is

intended for debugging purposes only.  The  -d  option,

when  used in conjunction with this option, can be very

effective in troubleshooting problems between the  SNPP

server and the remote paging terminal.

-l   Specify the service level for this page.   The  service

level  must  be  a  number between 0 and 11, inclusive.

The TME protocol specifies service level as follows:

0 - Priority

1 - Normal (default)

2 - Five minutes

3 - Fifteen minutes

4 - One hour

5 - Four hours

6 - Twelve hours

7 - Twenty Four hours

8 - Carrier specific '1'

9 - Carrier specific '2'

10 - Carrier specific '3'

11 - Carrier specific '4'

With the exception of level zero, service  levels  have

no  meaning to QuickPage but they are accepted for com-

patibility with other programs.  Pages  submitted  with

service  level  zero  cause  e-mail  to  be sent to the

Thomas Dwyer III    Last change: 21 June 1997                   2

User Commands                                        QuickPage(1)

submitter whenever a page succeeds or fails.  Any  ser-

vice  level  specified  by the user is passed on to the

SNPP  server.   This  option  applies   only   to   the

pagerid(s) specified by the next -p option.

-m   This option tells QuickPage to read an  e-mail  message

from  standard  input.   A  page is constructed by con-

catenating the From: header (XXX), the Subject:  header

(YYY),  and  lines  from the message body (ZZZ) as fol-

lows:

XXX (Subj: YYY) ZZZ ...

Minimal support is provided  for  multipart  MIME  mes-

sages.    The   first  part  with  a  Content-Type:  of

text/plain will be processed.  The remaining parts will

be discarded.  This will reduce the problems associated

with people  accidentally  mailing  binaries  to  their

pagers.

The X-sun-attachment type is not supported  and  should

not  be  used.   MIME is a documented standard.  The X-

sun-attachment type is neither documented nor standard.

However,  QuickPage  will  still use the From: and Sub-

ject: lines (but no message body)  from  such  messages

for those people who insist on using it anyway.

A line starting with two dashes  (--)  in  the  message

body  (or  in  a  MIME  text part) is assumed to be the

start of a signature and will cause QuickPage  to  stop

processing the message at that point.

-p   Specify the pagerid of the intended recipient.   Multi-

ple  recipients  may  be  specified by separating their

pager  IDs  with  commas,  slashes,  or  colons   (e.g.

pagerid1,pagerid2,pagerid3).   No  spaces are permitted

between recipients and separator characters.   Any  -a,

-c, or -l options previously encountered on the command

line are reset  to  their  default  values  after  this

option  is  processed.   This option may occur multiple

times on the command line, each  possibly  preceded  by

different -a, -c, or -l options.

-P   Like -p but does not reset -a, -c, or -l options previ-

ously encountered on the command line.

-q   Start a QuickPage daemon process.  This  option  causes

QuickPage  to detach itself from the controlling termi-

nal and create two processes: one to listen for  incom-

ing  SNPP  connections  and one for processing the page

queue.   This  option  also  specifies  the  number  of

seconds  QuickPage  sleeps between queue runs.  See the

Thomas Dwyer III    Last change: 21 June 1997                   3

User Commands                                        QuickPage(1)

NOTES section below for more information.

-Q   Print the current contents of  the  page  queue.   This

option may only be used on the QuickPage server.

-s   Specify the name(s) of the SNPP server(s).   The  host-

names  used  by a client to contact a server are deter-

mined by checking the following in this order:

the -s option

the environment variable SNPP_SERVER

the contents of the compiled in filename

the compiled in hostname

Multiple hostnames are  permitted  by  separating  them

with  commas  or colons.  If multiple servers are used,

it is assumed that they all have  identical  copies  of

the configuration file.

-v   Print the version of QuickPage and exit.

The QuickPage server requires a  configuration  file.   This

file is read once for each incoming SNPP connection and once

for each queue run.  The configuration file is  made  up  of

major  and  minor  keyword=value pairs.  Major keywords must

start at the beginning of a line.  Minor keywords must  con-

tain  leading  whitespace.  All keywords must be immediately

followed with an  equal  sign  (=).   Spaces  are  permitted

between  the  equal sign and the value.  Keywords may appear

in any order with the following three exceptions:

Minor keywords must be  grouped  together  under  their

respective major keyword.

Services must be defined before any pagers that  refer-

ence them.

Pagers must be defined before any groups that reference

them.

The major keywords are:

administrator  The  e-mail  address  of  the  QuickPage

administrator.    If   defined,   e-mail

notification will be sent to the  Quick-

Page administrator whenever a page fails

(i.e. when maxtries has been  exceeded).

This  address is also used in the Reply-

To: header for status notification  mes-

sages  sent  to  users  who submit high-

priority pages.

Thomas Dwyer III    Last change: 21 June 1997                   4

User Commands                                        QuickPage(1)

forcehostname  (true, false, or  @mailhost)  Force  the

destination address to be qualified with

a hostname when  sending  e-mail  status

notification  to users.  This option can

be used when  the  QuickPage  daemon  is

running  on a machine that does not han-

dle   unqualified    e-mail    addresses

correctly.  If the value of this keyword

starts with the '@' character,  it  will

be  appended as-is to unqualified e-mail

addresses.  If the value of this keyword

is  "true" then the submitter's hostname

will be appended to such addresses.  The

default  is  false  (do not append host-

names).

identtimeout   The number of  seconds  to  wait  for  a

reply   before  giving  up  on  RFC-1413

queries.  An integer less than or  equal

to  zero  disables  ident functionality.

The default is 10 seconds.

snpptimeout    The number of seconds  to  wait  for  an

SNPP command before terminating the con-

nection.  The default is 60 seconds.

queuedir       The location of the page  queue.   There

is  no  default  queue  directory;  this

option must be specified in  the  confi-

guration file.

service        The start of a  paging  service  defini-

tion.   The  argument to this keyword is

the name of the paging  service.   If  a

service named default exists, the values

of  its  minor  keywords  are  used   as

defaults  for  all other service defini-

tions.  This keyword has  the  following

minor keywords:

device         The name  of  the  device

the  modem  is physically

connected  to,  such   as

/dev/cua/a.   There is no

default   device;    this

option  must be specified

in   the    configuration

file.

dialcmd        The command, including  a

phone  number,  that  the

modem   should   use   to

Thomas Dwyer III    Last change: 21 June 1997                   5

User Commands                                        QuickPage(1)

connect   to  the  remote

paging service.  There is

no  default dial command;

this   option   must   be

specified  in  the confi-

guration file.

password       The password to use  when

logging  into  the remote

paging service.  The  IXO

specification defines the

password as  an  optional

six             character

alphanumeric string.  The

default   is   an   empty

string ("").

baudrate       The speed  to  use  while

talking   to  the  modem.

The default is 300 baud.

parity         The parity to use  (even,

odd, or none) while talk-

ing to  the  modem.   The

default is even.

maxmsgsize     The  maximum  number   of

characters  allowed  in a

single page segment.  The

default is 80 characters.

maxpages       The  maximum  number   of

page   segments  to  send

when  a  message  exceeds

maxmsgsize.  The value of

this   option   must   be

between    1    and    9,

inclusive.   The  default

is 9 page segments.

maxtries       The number  of  times  to

attempt  sending  a  page

before giving up.  If the

modem  is  busy (i.e. tip

or cu is currently  using

it)   or   if  the  modem

returns BUSY in  response

to  the dial command, the

counter  is  not   incre-

mented.    Thus   a  busy

modem will  cause  Quick-

Page  to  retry  the page

Thomas Dwyer III    Last change: 21 June 1997                   6

User Commands                                        QuickPage(1)

forever.  The default  is

6.

identfrom      (true or false) This key-

word specifies whether to

use the ident response as

the  message prefix if no

CALLerid SNPP command  is

received.  The default is

true.

allowpid       (true or false) This key-

word   specifies  whether

the QuickPage daemon will

accept  numeric  pagerids

for pagers not  specified

in    the   configuration

file.  The  default  ser-

vice  is  used  for  such

pagerids unless the  user

explicitly chooses a dif-

ferent   service.     The

default  for this keyword

is false.

pager          The start of a  pager  definition.   The

argument to this keyword is the username

associated with the pager.   This  user-

name  will be specified by the -p option

on the command line when running  Quick-

Page  in  client mode.  This keyword has

the following minor keywords:

pagerid        The pagerid sent  to  the

remote paging service for

this pager.

service        The  default  service  to

use for this pager.

group          The start of a  group  definition.   The

argument  to this keyword is the name of

a new page group.  This keyword has  the

following minor keyword:

member         A member of  this  group.

The  member  must  have a

valid pager entry  before

this   group  definition.

An optional duty schedule

(see below) may be speci-

fied.  This  keyword  may

Thomas Dwyer III    Last change: 21 June 1997                   7

User Commands                                        QuickPage(1)

appear   multiple   times

within a single group.

The default service, if present, must specify values for the

device  and  dialcmd  keywords.  If there is no default ser-

vice, all services must specify these values.

Member definitions within a page group have the syntax:

member=name[/DayStart-End]

Where  the  square  brackets  indicate  an   optional   duty

schedule.  The duty schedule has the same syntax as the Time

parameter in the UUCP Systems file:  Day is a list of  case-

sensitive  weekday abbreviations (e.g. MoTuTh), Start is the

start time (e.g. 800), and End is the end time (e.g.  1700).

The word Any is synonymous with SuMoTuWeThFrSa. Midnight may

be represented as either 0 or 2400.  The time range must not

span  across  midnight.  A slash is required to separate the

duty schedule from the member name.  Multiple member defini-

tions  for the same person with different duty schedules are

permitted  (see  the  example  below).    Overlapping   duty

schedules  for the same person within a group will not cause

duplicate pages to be sent to that person.  See the  follow-

ing example configuration file:

#

# QuickPage configuration file

#

identtimeout=10

queuedir=/var/spool/qpage

service=default

device=/dev/cua/b

dialcmd=ATDT482-1111

baudrate=1200

allowpid=yes

maxtries=6

# use the S7 modem register to set a connection timeout

service=supercom

dialcmd=ATS7=30DT482-9649

maxmsgsize=80

maxpages=9

pager=tomiii

pagerid=710

service=supercom

pager=tony

pagerid=711

service=supercom

Thomas Dwyer III    Last change: 21 June 1997                   8

User Commands                                        QuickPage(1)

group=sysadmin

member=tomiii/MoWeFr800-1700

member=tony/TuTh800-1700

member=tony/SaSu900-1600

NOTES

The order of the command line options is important.  The -a,

-c, and -l options must precede the pagerids they refer to.

The -p option resets -a, -c, and -l to their default values.

If this behavior is not desired, use -P instead.

The QuickPage daemon runs as two  separate  processes.   The

child  processes  the pages in the page queue.  If any pages

remain in the queue at the end of a run (if the remote modem

was  busy,  for  example, or if a page is being held until a

future time), the child sleeps for the time specified by the

-q  option  and  tries  again.   Otherwise the child goes to

sleep until awakened by the parent.

The parent daemon process listens for incoming SNPP  connec-

tions  and  forks an additional child process to handle each

request.  After a page is accepted, SIGUSR1 is sent  to  the

sleeping child process causing it to start a queue run.

If the CALLerid information received by the QuickPage daemon

contains  the '@' character, it is truncated at that charac-

ter before being prepended to messages.  However, it is used

as-is for the destination address when sending e-mail notif-

ication for high-priority recipients.

Due to limitations of SNPP, QuickPage has to  derive  e-mail

notification addresses from the CALLerid information.  Since

the CALLerid information might be bogus, all e-mail  notifi-

cations  are  sent  using  a  null  reverse path.  This will

prevent error messages from being generated by the mail sys-

tem if a bogus address is used for e-mail notification.

If the server does not receive a CALLerid command (i.e.  the

-f  flag  contains  a  null argument), notification messages

will not be sent, regardless of the specified service level.

When the -m flag is used to send a high-priority  page,  the

status  notification  is  sent  to the return address in the

original e-mail message unless overridden by the -f option.

The length of SNPP commands is limited only by the amount of

memory available to QuickPage.

QuickPage uses a timeout of 255 seconds while waiting for  a

connection  from the remote modem.  This allows the adminis-

trator to specify an  appropriate  timeout  by  setting  the

Thomas Dwyer III    Last change: 21 June 1997                   9

User Commands                                        QuickPage(1)

modem's S7 register in the dial command.

The modem must control the CD (carrier-detect) line;  other-

wise,  the  on/off hook status of the modem cannot be deter-

mined.  This is especially important if more than one paging

service  (phone  number)  is  used since QuickPage must know

when it's safe to send dial commands to the modem.

FILES

/etc/qpage.cf

SEE ALSO

RFC-1861

KNOWN BUGS

QuickPage cannot send a single e-mail  message  to  multiple

addresses.  If such functionality is desired, a mailing list

should be created for this purpose.

Pages are not queued on the client side.  As a result, if no

servers  are  available  to the client at the time a page is

submitted, an error message is printed and the page is  dis-

carded.

Please send bug reports to tomiii@mtu.edu.

AUTHOR

QuickPage was written by Thomas Dwyer  III  <tomiii@mtu.edu>

and is provided to the internet community free of charge.

Copyright (c) 1995-1997 Thomas Dwyer III

Thomas Dwyer III    Last change: 21 June 1997                  10